# Expansion Chamber Design

## Introduction

This document gives details of how to calculate the dimensions for both double and triple stage diffuser expansion chambers. All the information presented was taken from the books ‘The Basic Design of the Two Stroke Engine’ and the book ‘Design and Simulation of Two Stroke Engines’; both books are written by Professor G.P. Blair of Queens University Belfast, and published by the Society of Automotive Engineers. If you find this document interesting, you are well advised to read at least one of the books mentioned above, since they contain the author’s academic lifetime of knowledge on the two-stroke engine.

## Common Ground

Since both designs mentioned in this document rely on data obtained from engine parameters, these are discussed first.

### Speed of sound in a Gas (Pressure Wave Velocity)

One of the main parameters involved in expansion chamber design is the speed of sound, since this governs the speed of the pressure pulses that we use in the chamber.

| a0=γRTk | Where:- Tk is Exhaust gas temperature in Kelvin R is 287  is 1.4 a0 is in m/s |
| --- | --- |

### Engine State of Tune (Brake Mean Effective Pressure )

Of concern now is the determination of the exhaust gas temperature in Kelvin. This is usually a function of the engine’s state of tune or BMEP. This value BMEP for an engine is used in several of the expansion chamber design parameters, and is calculated as shown.

| BMEP=600000.kWSVCC.rpm | Where:- kW is engine power, kW (1bhp=746W) SVCC is swept volume, cc Rpm is engine speed, rpm BMEP is in Bar |
| --- | --- |

### Average Exhaust Temperature

Once the engine BMEP is determined, the exhaust average temperature can be calculated from the formula shown. This is an empirical measure based on readings taken during dyno run tests.

| Tk=534.67e0.0485BMEP | Where:- Tk is Exhaust gas temperature in K BMEP is in Bar |
| --- | --- |

This formula has been calculated from the following table.

| Bike | BMEP, Bar | Av. Exhaust Temp, C |
| --- | --- | --- |
| Grand Prix Racer | 11+ | 650 |
| Enduro | 8 | 500 |
| Roadster | 5 | 350 |

### Tuned Length of Exhaust

Blair’s formulae assume that the tuned length of the exhaust is to a point on the plugging cone, and is given by the formula below.

| Lt=1000a0Θep12rpm | Where:- Lt is tuned length, mm A0 is in m/s ep is exhaust duration, degrees |
| --- | --- |

### Effective Exhaust Diameter (EXD)

This is the diameter of a pipe whose area matches that of the exhaust port.

| EXD=4.width.heightΠ | Where:- EXD is effective diameter, mm Width is port width, mm Height is port height, mm |
| --- | --- |

### Exhaust Coefficients

There are several coefficients used in the design of the expansion chamber – these are a function of the engine’s state of tune.

| K0=1.0322e−0.0498BMEP | K1=1.9899e−0.0581BMEP |
| --- | --- |
| K2=0.08439e0.1226BMEP |  |

These formulae have been interpolated from the following table.

| Bike Style | BMEP, Bar | K0 | K1 | K2 |
| --- | --- | --- | --- | --- |
| Enduro | 8 | 0.7 | 1.125 | 2.25 |
| Motocross | 9-10 | 0.65 |  |  |
| GP Racer | 11+ | 0.6 | 1.05 | 3.25 |

## Two Stage Diffuser Expansion Chamber Dimension Calculation

A diagram of a typical two-stage diffuser expansion chamber is shown above. Note that the length of the plain pipe section LP01 includes the length of the exhaust port, i.e. LP01 is measured from the piston face.

### Dimension Calculation – Two Stage Diffuser

The following table gives the dimension for the two-stage diffuser expansion chamber section diameters.

| D1 = K1.EXD | D3 = K2.EXD | D4 = K0.EXD |
| --- | --- | --- |
|  | D2=D3LP12LP12+LP231.33 |  |

The next table gives the dimensions for the two-stage diffuser expansion chamber section lengths.

| LP01 = 0.10LT | LP12 = 0.41LT | LP23 = 0.14LT |
| --- | --- | --- |
| LP34 = 0.11LT | LP45 = 0.24LT | LP56 = LP45 |

## Three Stage Diffuser Expansion Chamber Dimension Calculation

A poorly drawn diagram of a typical three-stage diffuser expansion chamber is shown above. Note that the length of the plain pipe section LP01 includes the length of the exhaust port, i.e. LP01 is measured from the piston face.

### Dimension Calculation – Three Stage Diffuser

The following table gives the dimension for the two-stage diffuser expansion chamber section diameters.

| D1 = K1.EXD | D4 = K2.EXD | D5 = K0.EXD |
| --- | --- | --- |
| D2=D1eX12 | D3=D1eX13 |  |

Notice that two extra parameters are required for diameter calculation. These are given next.

X12=L12L12+L23+L34KhLogeD4D1

X13=L12+L23L12+L23+L34KhLogeD4D1

Notice also that an extra Coefficient has been introduced. This Coefficient Kh is called the horn coefficient, with typical values between one and two. Small values of Kh are best suited to GP engines with narrow power bands, larger values are for wider more flexible engines.

The next table gives the dimensions for the two-stage diffuser expansion chamber section lengths.

| LP01 = 0.10LT | LP12 = 0.275LT | LP23 = 0.183LT |
| --- | --- | --- |
| LP34 = 0.092LT | LP45 = 0.11LT | LP56 = 0.24LT |
| LP67 = LP56 |  |  |

## End Note

This document has provided formulae from the books mentioned in the introduction. The author cannot express how useful these books are for two-stroke design. All formulae are presented ‘as is’ with no warranty of suitability or correctness.

The author would be interested in hearing from any builder who uses the data presented here for construction of their own expansion chamber.

The author would also like to hear of other formulae currently in use for expansion chamber design.

The author would also like to receive constructive criticism and pointers for errors in the text of the document.